# MFIRE 3.0 Reference Manual

Overview     API Overview     GUI Overview

## MFIRE DLL API

### MFIRE Simulation Model Functions

The basis of the simulation of mine fires is the MFIRE Model. As described earlier, the Model consists of Airways, Junctions, Fans, and Fires. Each airway is associated with two junctions—a “starting” and “ending” junction based upon the airflow direction. Each Airway may also contain a single fan with a parameterized capacity. A series of connected airways make up a “Mesh,” and the Mfire model is made up of a series of Meshes, termed the “Mesh List.” At any time, within the simulation model (Mesh List), a single fire may exist and is associated with a specific airway.

Two mechanisms have been provided to facilitate creation of the Mesh List and subsequent initialization of the Model’s Event Queue. Models may be created from the Legacy Format text or the new XML based configuration file format. Once a model is created from either format, it can be modified and saved to an XML Configuration File.

Additionally, provisions have been made to obtain the MFireConfig class object directly and to create the model from a MFireConfig class object. These functions allow a user application to manipulate a copy of a simulation model directly and to use it to start a new simulation with MFire.

- LoadConfig (Text File, XML Config File)

- LoadConfig (XML Config Class ref)

- GetConfig

- SaveRunningConfig(XMLConfigFile)

### MFIRE XML Configuration File

The new style XML format configuration file can be created through the API. By loading a configuration, either from an old “legacy” format file and existing XML file, or by manually creating the configuration object, users can simply call the SaveRunningConfig() method to output the current configuration to an xml file.

### Callback Functions, Registration, and Deregistration

As described in the overview, the MFIRE DLL uses a callback interface to communicate with the user application, or GUI. Once registered with the DLL, callback functions are executed by the MFIRE Engine based upon operational events.

Any or all of the MFIRE DLL Callback functions can be subscribed to via registration, and there is no limit to the number of functions that can be registered to a specific callback mechanism. The list below itemizes the MFIRE DLL callback mechanisms.

#### Logger Callback

Provides messaging for Errors, Warnings Diagnostics, and Informational Events as the MFIRE Engine processes the Model Simulation. As the MFIRE Engine encounters conditions that are associated with a message, this callback is executed with the message severity and the message text being passed as parameters. The severity levels supported by MFIRE are the following:

- Fatal Error

- Warning

- Informational

- Debug

- Verbose Debug

#### Event Queue Callback

As events are encountered by the MFIRE Engine, they are processed and the MFIRE Model is updated accordingly. When registered, a user callback function will be executed with details of the specific event being passed as a parameter.

#### Simulator Tick Callback

Each cycle of the MFIRE Engine represents a Quantum of simulated time. The mathematical value of time represented by a quantum is provided to the MFIRE DLL as a configuration parameter during the creation of the model. The processor’s real-time clock is used to generate a “tick” at a configurable periodic rate. This clock tick causes the MFIRE Engine to execute one Quantum or Iteration of calculations. Upon completion, the Simulator tick callback, if registered, is executed with the simulation time being passed as a parameter.

The state of the model, represented by the results of the Quantum Calculations, may be output to the user application at a periodic rate, set as a configuration parameter during the creation of the model. The data is available as formatted and unformatted text. The formatted text is in the form of the classical report output of previous versions of MFIRE. Unformatted or Raw output is delimited text, more suited for parsing or importing into spreadsheet type applications.

#### Data Report (Unformatted Output) Callback

As each reporting period expired, the Unformatted output callback, if registered, is called, with the state of the model being passed as a string parameter. The output consist of delimited numerical data void of labels and comments.

#### Output (Formatted Output) Callback

As each reporting period expired, the Formatted output callback, if registered, is called, with the state of the model being passed as a string parameter. This output consist of formatted text, with column and row headers and additional informational comments in the style of the reports provided by previous generations of MFIRE.

#### Model Published Callback

The MFIRE simulation model allows access to internal data in the form of a dictionary structure (String, Value). As a ModelQueryEvent is executed, the Model Published callback, if registered, is called, with the model variables being passed as a dictionary parameter.

#### Monitor Published Callback

The MFIRE simulation model allows the user to monitor simulation variable values at the simulation “tick” rate. As each “tick” is executed, the Monitor Published callback, if registered, is called, with the model variables being passed as a dictionary parameter.

#### State Changed Callback

The MFIRE simulation model is controlled by the different states of execution. As the state of the simulation model changes, the State Changed callback, if registered, is called, with the state of the model being passed as a string parameter.

### Events And Event Queue

Execution of the MFIRE Engine may be altered by performing modifications on the MFIRE Model. Modifications may be requested to occur immediately or scheduled to be performed at a specific point in time in the Simulation. A chronological event queue has been implemented within the MFIRE DLL to manage scheduling and execution of events. At the start of each iteration of the MFIRE Engine, all events at the top of the event queue which have a timestamp equal to or earlier than the current simulation time are removed from the queue and processed.

#### Simulation Control Events

This series of events has been defined to provide a user with control of the overall execution of the Simulation in real time:

- AddPauseEvent

- AddContinueEvent

#### MFIRE Model Control Events

After an MFIRE Model has been created, the Model Control Events allow a user to modify or to schedule modifications to parameters within the Model. Each Event function is accompanied with a timestamp parameter to allow proper insertion into the chronological event queue. An event is processed by the MFIRE Engine immediately whenever the timestamp is less than the current simulated time, or equal to zero, regardless of the state of execution of the MFIRE Engine.

- AddAirwayNormalEvent

- AddChangeOutputTimeIntervalEvent

- AddChangeParameterEvent

- AddChangeTimeIncrementEvent

- AddClearFireEvent

- AddFanEvent

- AddFireSourceEvent

- AddModelQueryEvent

- AddShowDetailedJunctionOutputEvent

### MFIRE Model Queries

To provide the user interface with the ability to obtain information contained within the simulation model, a series of data query methods have been made available, as follows:

- GetAirwayNumbers

- GetConfig

- GetEngineState

- GetEventQueue

- GetInputUnits

- GetJunctionNumbers

- GetLogger

- GetOutput

- GetOutputUnits

- GetResumeTime

- GetSimulationSnapshot

- GetTime