# MFIRE 3.0 Reference Manual

Overview     API Overview     GUI Overview

## Overview

The MFIRE DLL is a Microsoft Windows Dynamic Link Library that provides tools that can be used to model and simulate mine fires. The MFIRE DLL is designed to interface to a user supplied Graphical User Interface application to present the calculation results of a simulation session. The user application (GUI) is used to present the data in whatever format is desired and to send the data to output devices and/or saved to data files.

To utilize the MFIRE DLL, the application executes calls to routines provided by the DLL to perform the following operations:

### Generate Model of Mine from a Configuration File

- Airways

- Junctions

- Fans

- Fire(s)

- Operational Parameters

### Register Callback functions to handle any of the available data streams

- Event Log Data

- Raw Calculation Data

- Formatted Calculation Data

- MFIRE Quantum Iteration Tick Notification

- Model Published Data

- Monitor Published Data

- Model State Changes

### Schedule Events

- Periodic or one-shot Output Events

- Operational Parameter Modification Events

### Execute Model Simulation

- Steady-State Initialization

- Non-Steady State simulation processing and output

- Quasi-Equilibrium simulation processing and output

Once Execution of the Model has been started, it will run asynchronously to the application within a separate thread called the MFIRE Engine. During execution of the Engine, additional events may be sent to the MFIRE DLL to modify the operation and output in real time. Events may be scheduled to occur immediately or at a specified time within the simulation. Examples of events are pause and restart events, the addition or modification of a fan to an airway, and modification of operational parameters.

The relationship between simulated time and actual time can be set to any desired value by changing the iteration delay parameter. A minimum value of 1 millisecond is enforced in order to prevent the MFIRE Engine from consuming excessive CPU resources. Each iteration results in a calculation cycle which represents a quantum of simulated time. The default quantum period is 15 seconds.

An example GUI Application has been provided with the MFIRE DLL as a reference. The example GUI has the capability of exercising all of the functional aspects of the MFIRE DLL.