## MFIRE User Guide

#### Version 3.0.20 (updated 2010-07-21)

## Menus

MFIRE can be navigated using menus with the keyboard by pressing the ALT key. A letter in the menu item name will become underlined. Pressing the underlined key will access the corresponding menu item.
Example: Pressing the ALT key, then the H key, then the A key, will access the MFIRE "About" information dialog.

### File menu

  - Open: Opens a configuration file.

  - Save: Saves the current configuration file as an .xml file (Prompts the user for a filename).
NOTE: If the current configuration was a legacy file (.txt extension) "Save As" will be 
invoked to save it as an XML file. Also see "Saving MFIRE Configurations" Section for more details.

  - Save As: Saves the current configuration file (Prompts the user for a filename and file type).
NOTE: See "Saving MFIRE Configurations" Section for more details.

NOTE: "Save" and "Save As" are only avaible when the Engine State is Stopped.

### Legacy/XML Converter menu

  - From Legacy Imperial:

    - To XML Imperial: Converts a legacy configuration file from imperial units to an XML configuration file in imperial units.

    - To XML Metric: Converts a legacy configuration file from imperial units to an XML configuration file in metric units.

  - From Legacy Metric:

    - To XML Imperial : Converts a legacy configuration file from metric units to an XML configuration file in imperial units.

    - To XML Metric: Converts a legacy configuration file from metric units to an XML configuration file in metric units.

  - From XML Imperial to XML Metric: Converts the units of an XML configuration file from imperial to metric.

  - From XML Metric to XML Imperial: Converts the units of an XML configuration file from metric to imperial.

### Simulation menu

  - Run: Runs the currently loaded configuration file.

  - End: Ends a simulation that is currently executing.

  - Pause/Continue: Pauses/Continues the configuration that is currently executing.
Note: While the simulation is paused the state of this item will change to "Continue" until either the "Continue" button is clicked or the "Continue" menu item is selected, changing the state back to "Pause."

### Options menu

  - Run Mode:

    - New: Runs MFIRE as a discrete event simulation (New Mode).

    - Legacy: Runs MFIRE in legacy mode.

  - New Mode Options:

    - Run Continuously: Runs the dynamic simulation continuously, executing the periodic events.
It effectively ignores the stop time set by the SPAN (legacy name)/TimeSpan (XML name) configuration variable.
Note: The simulation must be ended manually when this mode is enabled.

  - Input Units: Sets the simulation input units(Imperial,Metric).
Note: Loading an XML configuration file will automatically set the correct input units.

  - Output Units: Sets the simulation output units(Imperial,Metric).

### Help menu

  - Show Unit Conversions: Displays the MFIRE conversion table in both metric and imperial units.

  - MFIRE Reference Manual: Displays the MFIRE help document.

  - About: Displays the version information for the MFIRE application, MFire.dll and .Net Framework

## Control Panel

  - File: Displays the name of the configuration file currently open.

  - Open: Opens a configuration file for execution. Can be accessed via keyboard shortcut (Ctrl-O).

  - Run: Runs the dynamic simulation with the currently opened configuration file. Can be accessed via keyboard shortcut (Ctrl-R).

  - End: Ends the dynamic simulation. Can be accessed via keyboard shortcut (Ctrl-E).

  - Pause: Pauses the dynamic simulation.
Note: While the state of the simulation is paused, the title of this button will change to "Continue" until the "Continue" button is clicked, changing the state of the simulation to running and changing the caption back to "Pause."

## Time Group

- Displays the time properties of the dynamic simulation.

  - Time Scale: Provides the ability to scale the simulation execution speed based on the mode of operation.

    - Legacy mode: The default value is 1000.0(1000 times real-time processing). This cannot be changed in Legacy mode.

    - New mode: The default value is 1.000. This results in the simulation running in real time. This control has preset scaling values of 10.00, 5.00, 2.00, 1.00, 0.50, 0.20, and 0.10, but other values can be manually entered. Valid inputs are greater than 0.09. An invalid input will result in an error message being displayed and the default value being restored.
Note: Changes to the value of Time Scale will have an immediate effect on the dynamic simulation.

  - Model Time: Provides the elapsed simulation time(HH:mm:ss).

  - Real Time: Provides the system time(HH:mm:ss).

(Legacy mode) 


(New mode)

## Events

### Add Events Tab

- Allows the updating of configuration parameters, creation of new events, and changes to the dynamic simulation. Events may be added immediately, by providing a time that is less than the current Elapsed Time, and scheduled, by providing a time greater than the current elapsed time.
Note: If the Time parameter is not specified then the value 0.0 is assumed and the new event is executed immediately.

#### Note: The addition of events is available in New mode only.

#### Add Fire

This event edits the selected branch to a fire source, with the specified characteristics, at the "Time" specified.

Time: Event timestamp, Min. Note: Time is entered in an real format, ex. 1.5 equals 1 min 30 sec.
Airway: Available airways, loaded from the configuration file.
Contam. Flow Rate: Volume flow rate of contaminated gas inflow, Imperial Units are ft3/min, Metric Units are m3/sec.
Contam. Concentration: Concentration of contaminant in gas inflow, Percent.
Heat Input: Ref. Air Flow Rate defining fire characteristics, Imperial Units are Btu/min, Metric Units are Watts.
O2 Conc. Leaving Fire: Oxygen concentration of air current leaving fire zone, Percent.
Contam. Per Cu Ft O2: Contaminant production per ft3 of oxygen delivery, Imperial Units are ft3/ft3O2, Metric Units are m3/m3O2.
Heat Per Cu Ft O2: Heat production per ft3 of oxygen delivery, Imperial Units are BTU/ft3O2, Metric Units are kJ/m3O2.
Standard Air Flow: Heat entering or leaving airway, Imperial Units are ft3/min, Metric Units are m3/sec.
Transition Time: Leading time period for fire to reach full strength, Min.

#### Change Airway Resistance

This event changes the selected airway’s standard resistance equal to "Standard Resistance" at the "Time" specified.

Time: Event timestamp, Min. Note: Time is entered in a real format, ex. 1.5 equals 1 min 30 sec.
Airway: Available airways, loaded from the configuration file.
Standard Resistance: Standard resistance of airway, Imperial Units are 1.E-10 IN. W.G./(ft3)2, Metric Units are N*sec2/m8.

#### Change Output Time Interval

This event changes the time interval that data is output to the Verbose and Data tabs to the "New Interval" value specified, at the "Time" specified.

Time: Event timestamp, Min. Note: Time is entered in a real format, ex. 1.5 equals 1 min 30 sec.
New Interval: New Time interval value, Min. Note: New Interval must be an integer multiple of Time Increment and is entered in a real format, ex. 1.5 equals 1 min 30 sec.

#### Change Time Increment

This event changes the time increment in the dynamic simulation to the "New Increment" value specified at the "Time" specified. The time increment value specifies the rate at which the dynamic simulation performs data calculations.

Time: Event timestamp, Min. Note: Time is entered in a real format, ex. 1.5 equals 1 min 30 sec.
New Increment: New Time increment value, Sec. Note: New Increment is entered in a real format, ex. 90.0 equals 1 min 30 sec.

#### Edit Fan

This event edits the selected airway to an airway with a fan, with the specified characteristics, at the "Time" specified. If the selected airway contains a fan then the Airflow and Pressure values will be populated with the fan data; otherwise the Airflow and Pressure values are populated with the default value of zero.
NOTE: A fan can contain between two and five points of data any less and the remaining values are populated with the default value of zero.

Time: Event timestamp, Min. Note: Time is entered in an real format, ex. 1.5 equals 1 min 30 sec.
Airway: Available airways, loaded from the configuration file.
Use Data: Enables the Airflow and Pressure data input set.
Airflow: Airflow Rate, Imperial Units are ft3/min, Metric Units are m3/sec.
Pressure: Corresponding fan pressure, Imperial Units are IN. W.G., Metric Units are Pascals.

#### Query Simulation Model

This event forces the printing of the dynamic simulation configuration values to the model tree, on the model tab, starting at "Time" Min. and occurring at the interval designated by "New Interval" Min. from the queue.
Note: A single query can be performed by setting the "New Interval" field to zero or by leaving it blank.
Note: The dynamic simulation is paused each time this event is processed requiring the "Continue" button be clicked to resume simulation processing.

Time: Event timestamp, Min. Note: Time is entered in an real format, ex. 1.5 equals 1 min 30 sec.
New Interval: Rate at which to perform the simulation model query, Min. Note: New Interval is entered in an real format, ex. 1.5 equals 1 min 30 sec.

#### Remove Fire

This event changes fire branch to an ordinary airway at the "Time" specified.

Time: Event timestamp, Min. Note: Time is entered in an real format, ex. 1.5 equals 1 min 30 sec.
Airway: Available airways, loaded from the configuration file.

### Event Queue Tab

- Displays the events currently scheduled in the dynamic simulation queue by both "Timestamp" and "Event Type" and can be sorted by both categories. An individual event can be removed from the display by clicking the "Remove Event" button, or all events can be removed from the display by clicking the "Clear All Events" button; this also removes them from the event queue. As the event queue is processed by the dynamic simulation, the event is removed from the display.

### Model Tab

- Displays the dynamic simulation input parameters and the associated value. When used in conjunction with the Query Simulation Model event, the parameter values can be monitored while the simulation is running. The "Last Updated" display below the model tree indicates when, in elapsed time, the parameters in the model tree were last updated. Each parameter in the path leading to an updated value is indicated by a red "!".

#### Note: The Monitor List feature is available in New mode only.

- Values are added to the Monitor List by selecting them from the Model tree. This is done by right-clicking on a parameter value and accessing the context menu.

Add to Monitor: Adds the selected value to the Monitor List. If the value is already on the monitor list this option will not be available.
Remove from Monitor: Removes the selected value from the Monitor List. If the value is not on the monitor list this option will not be available.
Clear All from Monitor: Removes all values from the Monitor List. If the monitor list is empty this option will not be available.

NOTE: Changes made to the Monitor List occur immediately.
NOTE: The Monitor List will be cleared each time a new configuration is loaded.

## Data Presentation

- Currently four MFIRE output types are available: Log, Verbose, Data, and Monitor. Each has the option to save the current output to file by clicking the "Save As" button. Each has the option to clear the current output by clicking the "Clear" button.
Note: The current output is cleared each time the dynamic simulation "Run" button is clicked.

### Log

- Displays log messages while the dynamic simulation is executing. The current output can be filtered by selecting a message type from the "Diagnostic Level" drop-down menu. This output is not automatically saved.

Current Diagnostic Levels include:

  - ALL (Messages of all types are listed).

  - DEBUG (Messages used to debug the dynamic simulation).

  - ERROR (Messages indicating an error condition has occurred in the data file, results in program termination).

  - FATAL (Messages indicating a fault has occurred in the dynamic simulation).

  - INFO (Messages reporting the current status of the dynamic simulation).

  - WARNING (Messages indicating a possibly wrong value and/or its calculated intermediate result is detected in the data file; results in program continuation).

### Verbose

- Displays the detailed output from the dynamic simulation. This output is saved automatically in the "Application_Directory\Simulations" directory and the files have the following naming convention "Simulation_yyyyMMdd_HHmmss.txt". Where yyyy = year, MM = month, dd = day, HH = Hour (24 hour format), mm = minutes, ss = seconds.

Note: Output is always presented in imperial units.

### Data

- Displays the raw output from the dynamic simulation. The raw output can be saved automatically by checking the "Auto Save Files" checkbox; data is saved at the rate indicated by TimeIncrement(TINC). The data is located in the "Application_Directory\DataReports" directory and the files have the following naming convention "DataReport_yyyyMMdd_HHmmss.txt". Where yyyy = year, MM = month, dd = day, HH = Hour (24 hour format), mm = minutes, ss = seconds.

Note: Output will be presented in either imperial or metric units based on what is selected.

#### Note: The Monitor List feature is available in New mode only.

### Monitor

- Displays the selected values at the rate indicated by TimeIncrement(TINC) while the dynamic simulation is running. This output is not automatically saved.
NOTE: See the "Monitor Tab" section for adding values to the Monitor List.
NOTE: A large number of values added to the monitor will have an impact on Simulation performance.

### Snapshot

- A fifth output, not presented on the MFIRE interface, is the Model configuration snapshot. The snapshot is taken once when the configuration is initially loaded, or if the dynamic simulation is run multiple times then the snapshot is taken when the "Run" button is clicked and again when the execution of the dynamic simulation is completed. The snapshot is located in the "Application_Directory\Snapshots" directory and the files have the following naming convention "Model_yyyyMMdd_HHmmss.txt". Where yyyy = year, MM = month, dd = day, HH = Hour (24 hour format), mm = minutes, ss = seconds.
Note: When used in conjunction with the Query Simulation Model event, a new snapshot is created each time a Query Simulation Model event is processed by the dynamic simulation.

## Status Bar

- Displays the current status of the dynamic simulation, along with unit types, brief configuration information, and application mode.

Current status messages:

  - Engine State(STOPPED, RUNNING, PAUSED, COMPLETED).

  - Units(IN(Imperial or Metric), OUT(Imperial or Metric)).

  - Model Status(# Airways, # Fans, # Junctions, # Fires).

  - Run Mode(Legacy, New(Real-time or Delayed,Continuous or Fixed Timespan)).

(Shows a STOPPED simulation with input Units of Imperial and output Units of Metric, the configuration contains 53 Airways, 2 Fans, 34 Junctions and 2 Fires, in Run Mode Legacy) 


(Shows a PAUSED simulation with input and output Units of Imperial, the configuration contains 44 Airways, 2 Fans, 35 Junctions and 1 Fire, in Run Mode New(Delayed, Continuous))

### Running MFIRE Simulations

  - The first step is to load an MFIRE configuration: legacy text or XML format. Utilize the Ctrl-O shortcut key to bring up the "Open Configuration" dialog.

  - The next step is to select the Run Mode options. By default, the application starts up in Legacy Run Mode. This means the simulation will be run as fast as possible till the end of the simulation. The simulation endtime is dictated by the SPAN (legacy name) or TimeSpan (XML name) configuration variable.

To enable "New" Run Mode, navigate to the Options->Run Mode menu and select the "New" option. This will enable the "New Mode Options" menu. You now have the option to run the simulation without a time boundary or continuously. In addition, you also have the option to run the simulation in real time. In New Mode, you may select any combination of these options. The currently set Run Mode options are displayed in the application’s status bar.

  - Once a configuration has been loaded and the run options have been set, simply click the "Run" button (Ctrl-R) to begin the simulation.

### Converting Imperial To/From Metric

  - A Legacy or New Mode configuration file can be converted to Imperial or Metric XML using the "Legacy/XML Converter" menu options. Simply select the menu item that best describes your needs. For example, if you want to convert metric Legacy file to Imperial XML, select "From Legacy Metric->To XML Metric." You will be prompted for the source and destination files accordingly.

### Saving MFIRE Configurations as XML

  - When a Legacy configuration file is loaded, it may be saved as a New Mode XML file. Simply navigate to the File menu and select "Save."

### Saving MFIRE Configurations

  - MFire can save the current configuration at any time(in XML format).

    - To save the configuration that is currently executing, click Pause and then End. The "File" menu "Save" and "Save As" options are then made available. All events that have been added since the loading of the configuration file will be saved. When this type of saved configuration is reloaded the simulation will execute at Legacy mode speed, pausing at the time when the configuration was saved. Upon continuing, the simulation will execute at the defined New mode speed.

    - Save a configuration that has completed by clicking the "Save" or "Save As" option under the "File" menu. This will store all events that have been added since the loading of the configuration file. When this type of saved configuration is reloaded the simulation will execute at the defined New mode speed.

### Using the Monitor List

  - Load a configuration file.

  - Switch to "New" mode.

  - Select the "Model" tab.

  - Right-click a variable value to access the monitor context menu. To add a value to the monitor, select "Add to monitor" from the context menu. If the value is already in the monitor this option will not be available. To remove a value from the monitor list, select "Remove from monitor" from the context menu. If the selected value is not in the monitor this option will not be available. To remove all values from the monitor the user will select "Clear All from monitor" from the context menu. If the monitor is empty this option will not be available.

  - Selected values will be printed, at the rate of TimeIncrement, in the Monitor tab output window while the simulation is running.

  - Save the output of the Monitor tab by clicking the "Save As" button or clear the output by clicking the "Clear" button.