Control Card 1

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| NumOfAirways | NB |  | None |
| NumOfFans | NFNUM |  | None |
| NumOfFires | INFLOW |  | None |
| OmitJunctions | NVPN | ‘0’ or ‘1’ | None |
| NetworkOnly | NETW | ‘0’ or ‘1’ | None |
| ReferenceTemperature | TR | >=10 Deg. F; <=110 Deg.F | 75 Deg.F Note 1 |
| MaxTemperatureIterations | MADJ | >= 5; <=80 | 10 Note 1 |
| MaxDynamicIterations | ITN | >=5; <=80 | 10 Note 1 |
| TemperatureOnly | NTEMP | ‘0’ or ‘1’ | None |
| TimeIncrement | TINC | >0 | 15 seconds Note 1 |
| TimeSpan | SPAN | >0 | None |
| OutputDetail | IOUT | 1 = Normal report 0 = Brief report -1 = Detailed report --2 = More detailed report | None |
| TimeIntervalForReport | TOUT |  |  |
| TransientState | CONCT | ‘0’ or ‘1’ |  |
| ReferenceDensity | DR |  | Note 2 |

Note 1:  In the current version of MFIRE, an error message is displayed if these parameters are omitted or outside the acceptable limits.  Insert the appropriate default value into the input file to replicate the default value substitution of the previous version of MFIRE.

Note 2:  Default calculated based on reference temperature.  If specified, the ratio of the specified reference density to the calculated reference density must be less than 0.7.

Airway Cards

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| Number | NO | >=1; <9990 | None |
| StartJunction | JS | >=1; <=999 | None |
| EndJunction | JF | >=1; <=999 | None |
| Type | NWTYP | 0 = Normal airway 1 = Fan airway -1 = Fixed quantity airway 10 = Airway contains fire source | None |
| Resistance | R | > 0 | Note 1 |
| FlowRate | Q | >= 1000 | Note 2 |
| FrictionFactor | KF | >= 10.0; <= 1000.0 | 100.0 Note 3 |
| Length | LA | >= 10.0; <= 3000.0 | 500.0 Note 3 |
| CrossSectionalArea | A | >= 0.1; <= 500.0 | 100.0 Note3, Note 4 |
| Perimeter | O | >= 0.1; <= 100.0 | 40.0 Note3, Note 5 |
| ThermalDefusivity | HA | >= 1.0E-5; <= 10.0 | 0.1 Note 3 |
| ThermalConductivity | HK | >= 1.0E-5; <= 50.0 | 3.0 Note 3 |
| CH4EmissionRateAirway | CH4V | None | None |
| CH4EmissionRateSurfArea | CH4PA | None | None |
| RockTemperature | TROCK | None | None |

Note1:  If the specified resistance is less than or equal to zero for a normal airway (type = 0), the resistance is calculated based on the Friction Factor, Length, Cross-sectional Area, Perimeter, and Reference Density.  In fan airways that do not calculate pressure based on a fan curve (fix flow), the resistance represents the pressure in the airway.

Note 2:  If the initial airway flow rate is less than 1000, the flow rate is set to 1000.

Note 3:  If the value for these parameters is omitted or if the value specified is outside of the acceptable range, the parameter is set to the average default value.  If average values are not specified in the network specification, the above default values are used.

Note 4:  If a value of less than or equal to 0.01 is specified initially (before default substitution) for the cross-sectional area and the value of the perimeter is within valid limits, the value of the cross-sectional area will be calculated based on the value provided by the perimeter.  If the value of the perimeter is not within limits, an error message will be displayed.

Note 5:  If a value of less than or equal to 0.01 is specified initially (before default substitution) for the perimeter and the value of the cross-sectional area is within valid limits, the value of the perimeter will be calculated based on the value provided by the cross-sectional area.  If the value of the cross-sectional area is not within limits, an error message will be displayed.

Junction Cards

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| Number | JNO | >=1; <999 | None |
| Temperature | T | >= 35; <= 125 | None |
| Elevation | Z | None | None |
| CH4InitialConc | CH4C | >= 0; <= 100 | None |

Fan Card 1

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| AirwayNo | NOF | Note 1 | None |
| DataCount | MPTS | >= 2; <= 10 | None |
| CurveFittingMethod | NSWT | 1 = Least Square 2 = Spline 3 = Auto select | None |

Note 1:  The airway number specified must exist in the mine structure of the fan data will be flagged as isolated.

Fan Card 2

Air flow data and pressure data occur as paired data items (up to five pairs per card; maximum total of ten data pairs)

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| AirFlowData | QF | >= 0; Note 1 | None |
| PressureData | PF |  |  |

Note 1:  Air flow data must be specified in increasing order of magnitude.

Fan Card 3

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| FanCurveBoundry | NPLOT | 1 = Curve is extended following its gradient at both ends; 2 = Left boundary extended following gradient, right boundary is set to zero; 3 = Both boundaries are sent to zero | None |

Contamination (Fire) Card

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| AirwayNo | NCENT | Note 1 | None |
| ContamFlowRate | CONT | >= 0 | None |
| ContamConcentration | CONC | >= 0; <= 100 | None |
| HeatInput | HEAT | Note 2; Note 3 | None |
| O2ConcLeavingFire | O2MIN | >= 0 | None |
| ContamPerCuFtO2 | SMPO2 | >= 0 | None |
| HeatPerCuFtO2 | HTPO2 | >= 0 | None |
| StandardAirFlow | QCENT | >= 0; Note 2 | None |
| TransitionTime | TPR | >= 1.0E-5 | None |

Note 1:  The airway number specified must exist in the mine structure.

Note 2:  The heat input and standard air flow are limited to prevent abnormal temperature output from the fire source.

Note 3:  If a negative heat input is specified, the contamination source is treated as a fix capacity cooling station.

Control Card 2

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| AvgValueDataPresent | NAV | <= 0 – Average value inputs have been omitted >= 1 – Average value inputs present | None |
| StartJunction | JSTART | Note 1 | None |
| StartJunctionTemperature | TSTART | >= -40; <= 120 | None |
| TimeEquillibrium | TIME | > 0 | 10.0 Note 2 |
| FumeCriteria | CRITSM | >= 1.0E-4; <= 1.0 | 0.005 Note 2 |
| CH4Criteria | CRITGS | >= 1.0E-4;; <= 1.0 | 0.01 Note 2 |
| TemperatureCriteria | CRITHT | >= 1.0E-4; <= 1.0 | 0.1 Note 2 |
| PressureDropWarningLimit | WRNPR | >= 1.0E-10; <= 0.5 | 0.01 Note 2 |
| FumeWarningLimit | WRNSM | >= 1.0E-10; <= 1.0 | 0.05 Note 2 |
| CH4WarningLimit | WRNGS | >= 1.0E-10; <= 10.0 | 1.0 Note 2 |
| TemperatureLimit | WRNHT | >= 1.0E-10; <= 1000.0 | 100.0 Note 2 |

Note 1:  The airway number specified must exist in the mine structure.

Note 2:  In the current version of MFIRE, an error message is displayed if these parameters are omitted or outside the acceptable limits.  Insert the appropriate default value into the input file to replicate the default value substitution of the previous version of MFIRE.

Average Value Card

| New Parameter Name | Old Parameter Name | Limits | Default |
| --- | --- | --- | --- |
| AverageDiffusivity | HAAVR | >= 1.0E-5; <= 10.0; | 0.1 Note 1 |
| AverageConductivity | HKAVR | >= 1.0E-5; <= 50.0 | 3.0 Note 1 |
| AverageFrictionFactor | KFAVR | >= 10.0; <= 1000.0 | 100.0 Note 1 |
| AverageLength | LAAVR | >= 10.0; <= 3000.0 | 500.0 Note 1 |
| AverageCrossSection | AAVR | >= 0.1 <= 500.0 | 100.0 Note 1 |
| AveragePerimiter | OAVR | >= 0.1 <= 100.0 | 40.0 Note 1 |

Note 1:              Default values are loaded into these parameters if an out or range input is specified or if the average value inputs are omitted (AvgValueDataPresent <= 0).