MFIRE 4.0 is an upgraded version of MFIRE 3.0, with major improvements made over the previous version. The main objective of this document is to briefly introduce the technical improvements of MFIRE 4.0 and to provide tutorials to MFIRE users. The targeted users of this document are those experienced users of MFIRE 3.0 or any other previous versions.

What is new in MFIRE 4.0

- Added a time-dependent fire source: t-squared fire

- Added the ability to input a heat release rate (HRR) curve from a text file

- Added the ability to identify the occurrence of smoke rollback in a fire branch

New fire sources added in MFIRE 4.0

Besides the three fire source models—fixed-heat-input fire, oxygen-rich fire, and fuel-rich fire—which MFIRE 4.0 inherits from MFIRE 3.0, two time-dependent fire models—t-squared fire and HRR curve input fire from a file—are added to MFIRE 4.0.

- t-squared fire

In a t-squared fire, the HRR varies proportionally to the square of time. In general, fire development varies depending on the combustion characteristics of the fuel(s), the physical configuration of the fuel(s), and the availability of combustion air. Once a stable flame is attained, most fires grow in an accelerating pattern, reach a steady state characterized by a maximum HRR, and then enter into a decay period as the availability of either fuel or combustion air becomes limited. The overall fire development process can be divided into growth period, fully developed period (or steady period), and decay period. In the growth period and decay period, the HRRs of a fire are approximately proportional to the square of time. Therefore, a t-squared fire can be described with three segments representing the three periods: an increasing HRR during the fire growth period (from t0 to t1), a simplified constant HRR for the fully developed fire period (from t1 to t2), and a declining HRR for the decay period (from t2 to t3). The time period from 0 to t0 is called the ignition delay period which is the period from ignition to flaming. It is assumed that there is no heat release during this period.

Figure 1.

In MFIRE, the required inputs of a t-squared fire are the time of each period and the peak value of HRR. With the parameters input by users, the following segmented equation is applied to interpret the HRR of all the periods of a fire.

where:

heat release rate (Btu/min)

growing factor of growth period (Btu/min2)

decaying factor of decay period (Btu/min2)

maximum heat release rate (Btu/min)

t 	        time (min)

t0 	        time of ignition delay (min)

t1 	        the start time of steady fire period (min)

t2 	        the end time of steady fire period (min)

t3 	        the end time of decay period (min)

In order to specify a t-squared fire in MFIRE, users are required to enter all the time periods and the peak HRR value. The growing factor () and the decaying factor () can be determined by the following equations

It should be noted that it is not necessary for the growing factor and the decaying factor to be the same in a fire scenario. It is very common that fires grow much faster than decay showing a larger growing factor than the decaying factor in a fire.

- HRR curve input from file

If users have a detailed HRR curve data saved in a file, there is no need to modify it to fit the current fixed-heat-input fire, oxygen-rich fire, fuel-rich fire, or t-squared fire. MFIRE 4.0 is able to read an external data file to enter the fire model. For example, Figure 2 displays an HRR curve obtained from a conveyor belt fire test conducted at the National Institute for Occupational Safety and Health (NIOSH). Although the HRR curve can be simplified and fit t-squared fire can, this simplification definitely will reduce the precision of the fire data. With the feature of the HRR curve input from file, MFIRE 4.0 is able to enter exact HRR with time without any simplification.

Figure 2. Identification of smoke rollback phenomena.

Smoke rollback occurs in tunnels when the buoyancy force generated by a fire overcomes the inertial forces of ventilation to cause smoke migration upwind along the roof counter to the positive ventilation. Smoke rollback can be a dangerous and potentially fatal threat to miners and firefighters in an underground mine fire, preventing firefighters from getting close enough to fight a fire effectively in an underground mine entry. It can also bring flame from the fire back onto the firefighters. In addition, the heat from the rolling smoke from a fire occurring in a return airway may destroy stoppings used to separate intake and return airways. Subsequently, the smoke and products of combustion (POC) can contaminate the intake fresh air and be brought to the active faces where miners are working or evacuating.

MFIRE 4.0 applies critical velocity to determine the occurrence of a smoke rollback in a fire branch. Critical velocity is the minimum airflow velocity preventing the smoke from rolling back. It can be calculated by the following equation:

where

Cp			specific heat of air at constant pressure, J/(kg,k)

g			acceleration due to gravity, m/s2

hydraulic height of tunnel, m

heat release rate, kW

dimensionless heat release rate

T0			temperature of the approach air, K

V			velocity of the approach air, m/s

dimensionless maximum airflow velocity

Vc			critical velocity of smoke rollback, m/s

entry slope in degrees

The dimensionless HRR is determined as below:

The smoke rollback length Lr is estimated using the following equation:

where

A            cross-sectional area of tunnel, m2

Tf            average temperature of the fire-site gases, K

Vin          velocity of the approach air, m/s

Format input files for MFIRE 4.0

- Input t-squared fire

MFIRE 4.0 accepts two formats of input file: text file and xml file. Users have two options to enter a t-squared fire in both the text file and the xml file.  ne is from the Fire Line and the other is from the Event Change Line. A t-squared fire requires the following five parameters entered in the input file:

TLEAD: t0, time of ignition delay (min)

TMAX: t1, the start time of steady period (min)

TSTEADY: t2, the end time of steady period (min)

TDECAY: t3, the end time of decay period (min)

QMAX: , maximum heat release rate (Btu/min)

FUMEPROCONSTANT: CO/CO2 production rate constant (ft3/min)

Note: The values of FumeProConstant for different combustible materials can be found at Appendix 1 Table 2 in “MFIRE User’s Manual Version 2.20.”

Figure 3 shows an example that a measured heat release rate of a diesel fire. To enter the heat release rate to MFIRE, a t-squared fire can represent this diesel fire very closely with the listed parameters in Table 1. The CO production rate constant can use the value of 0.00009 ft3/Btu.

Figure 3.

Table 1. t-squared fire input parameters

| Peak HRR, Btu/min | 1061176 |
| --- | --- |
| Time of ignition delay (t0),min | 2 |
| Start time of steady period (t1),min | 5 |
| End time of steady period (t2),min | 8 |
| End time of decay period (t3),min | 25 |

Enter the t-squared fire in the fire line of text file

Changes have to be made in the fire source line of MFIRE input file to enter a t-squared fire. Six parameters specifying a t-squared fire and a new FIRETYPE variable are added to the fire line. The variable FIRETYPE is used to flag the type of fire sources: 1- HRR curve input fire; 2-t-squared fire; 3-MFIRE 3.0 fires.

|  |  | MFIRE 3.0 fire sources |  |  |  |  |  |  |  | t-squared fire |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NCENT | FIRETYPE | CONT | CONC | HEAT | O2MIN | SMPO2 | HTPO2 | QCENT | TPR | TLEAD | TMAX | TSTEADY | TDECAY | QMAX | FUMEPROCONSTANT |

To enter the above t-squared fire example in Airway #42, the fire line is formatted as below:

42, 2, 0, 0, 0, 0, 0, 0, 0, 0, 2, 5, 8, 25, 1061176, 0.0009

Enter the t-squared fire in the fire line of xml file.

The following is the xml script for the above t-squared fire in an xml file:

- <Fires>

- <Fire>

<AirwayNo>42</AirwayNo>

<ContamFlowRate>0</ContamFlowRate>

<ContamConcentration>0</ContamConcentration>

<HeatInput>0</HeatInput>

<O2ConcLeavingFire>0</O2ConcLeavingFire>

<ContamPerCuFtO2>0</ContamPerCuFtO2>

<HeatPerCuFtO2>0</HeatPerCuFtO2>

<StandardAirFlow>0</StandardAirFlow>

<TransitionTime>1E-05</TransitionTime>

<FireType>2</FireType>

<Tlead>2</Tlead>

<Tmax>5</Tmax>

<Tsteady>8</Tsteady>

<Tdecay>25</Tdecay>

<Qmax>1061176</Qmax>

<FumeProConstant>0.0009</FumeProConstant>

<HeatReleaseData />

<HasSmokeRollback>false</HasSmokeRollback>

<LengthOfRollback>0</LengthOfRollback>

</Fire>

</Fires>

Enter the t-squared fire in the timetable line of text file.

It is known that condition changes during a fire simulation can be made in MFIRE through the timetable line. New fire can be entered by changing a regular branch to a fire source branch with fire characteristics with ICODE=3.

For example, if the above t-squared fire source is entered to Airway #42 at time of 3 minutes, the following line need to be formatted:

|  |  |  | MFIRE 3.0 fire sources |  |  |  |  |  |  |  | t-squared fire |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TS | ICODE | NBR | CONT | CONC | HEAT | O2MIN | SMPO2 | HTPO2 | QCENT | TPR | TLEAD | TMAX | TSTEADY | TDECAY | QMAX | FUMEPROCONSTANT |

3, 3, 42, 0, 0, 0, 0, 0, 0, 0, 0, 2, 5, 8, 25, 1061176, 0.0009

Enter the t-squared fire in the timetable line of xml file.

The following is the xml script for the above t-squared fire in an xml file:

- <FireSourceEvents>

- <FireSourceEvent>

<EventType>3</EventType>

<Periodic>false</Periodic>

<RecurringInterval>0</RecurringInterval>

<TS>3</TS>

<AirwayNo>42</AirwayNo>

<ContamFlowRate>0</ContamFlowRate>

<ContamConcentration>0</ContamConcentration>

<HeatInput>0</HeatInput>

<O2ConcLeavingFire>0</O2ConcLeavingFire>

<ContamPerCuFtO2>0</ContamPerCuFtO2>

<HeatPerCuFtO2>0</HeatPerCuFtO2>

<StandardAirFlow>0</StandardAirFlow>

<TransitionTime>0</TransitionTime>

<Tlead>2</Tlead>

<Tmax>5</Tmax>

<Tsteady>8</Tsteady>

<Tdecay>25</Tdecay>

<Qmax>1061176</Qmax>

<FumeProConstant>0.0009</FumeProConstant>

</FireSourceEvent>

</FireSourceEvents>

Enter the t-squared fire via interface in TestApp.

Open an input file and then choose Options -> Run Mode -> New from the menus. At the Add Events tab, select AddFire at Select Event.

- HRR curve input file fire

The HRR curve input file fire can only be entered via the fire source line in MFIRE text input file. This fire type requires a preformatted text file containing two columns with one column recording time and the other recording HRR. Simply specify FIRETYPE=1 in the fire source line, and a window will be brought up to read the HRR input file.

For example, enter the fire line below in the MFIRE input file:

42,1,0,0, , , , , ,

After opening the input file, the following window will open up to allow the user to select the HRR input file.

- Smoke rollback identification

There is no input requirement for users for smoke rollback identification. A warning message regarding the occurrence of smoke rollback and rollback length will be included in the MFIRE output files.